SIMATS ENGINEERING

CO3- AT2 - Design Task - Medium

COURSE NAME: Advance Wireless Communication Systems /

5G / 6G Communication Systems

COURSE CODE: ECA5604 Slot-A

COURSE OUTCOME COVERED

CO: 3 - Design spectrum access strategies, waveform generation methods, and

multiple access techniques for efficient wireless transmission systems. (BL6)

Assessment Tool Number : CO3_AT2

Assessment Tool Weightage : 35%

Assessment Tool : Design Task

Duration : 180 Minutes

Marks : 100 Marks

Type of Assessment Conduction : At Home

Evaluation Type : GitHub

Result Generation : Students design and justify appropriate wireless

communication solutions by applying engineering

principles to meet specified requirements.

Topic Covered :

Spectrum management, licensed/unlicensed spectrum, dynamic spectrum access,

cognitive radio, OFDMA, resource block allocation, GFDMA, NOMA, power- domain

multiplexing, SIC, and IoT applications. Advanced waveforms and wireless

technologies including GFDM, FBMC, UFMC, mmWave propagation, path loss and

penetration, millimeter-wave antenna systems, MIMO, and Massive MIMO

fundamentals.

Short description about assessment: Students design a system / model based on

given requirements to assess creativity and application skills. This question paper

consists of a total of 40 questions, out of which each student is required to answer any

2 questions. Each question carries 50 marks, and the paper carries a total of 100

Marks

QUESTIONS: 2 (2 X 50 = 100)                 Total Marks: 100 Duration: 3hrs

- Design a low-power communication model for battery-operated IoT devices. (BL6)

- Design an NB-IoT architecture for smart utility monitoring. (BL6)

Code of Conduct:

I __Velamuri Bhanodhay (192412330)___ (Name / Reg No) certify that this submission is my original  work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary  action.

Signature of the student:

RUBRICS FOR CALCULATION:

Design Task

| Course Code & Title: | ECA5604 -5G/6G COMMUNICATION | Course Faculty: | Dr.K.Sathish |
| --- | --- | --- | --- |
| Student Name: | Velamuri Bhanodhay | Reg. No.: | 192412330 |
| Dept / Batch: | BE.ECE / 2024 | Date: | 10/8/2026 |

- Design a low-power communication model for battery-operated IoT devices.

Idea: Use sleep mode + short data packets + low transmission power to reduce battery consumption.

Program Code:-

clc; clear; close all;

% IoT communication parameters

N = 100;                 % Number of transmissions

Ptx = 20;                % TX power (mW)

Psleep = 0.1;            % Sleep power (mW)

Ttx = 0.01;              % Transmission time (s)

Tframe = 1;              % Frame duration (s)

% Energy consumed per frame

Etx = Ptx*Ttx;

Esleep = Psleep*(Tframe-Ttx);

Eframe = Etx + Esleep;

% Total energy

Etotal = N*Eframe;

fprintf('Energy per frame = %.3f mJ\n',Eframe);

fprintf('Total energy = %.3f mJ\n',Etotal);

% Compare with always-active communication

Eactive = N*Ptx*Tframe;

fprintf('Always-active energy = %.3f mJ\n',Eactive);

% Energy saving

saving = (1-Etotal/Eactive)*100;

fprintf('Energy saving = %.2f %%\n',saving);

% Plot

bar([Etotal Eactive])

set(gca,'XTickLabel',{'Low-Power IoT','Always Active'})

ylabel('Energy (mJ)')

title('Energy Consumption Comparison')

grid on

Design block diagram

Sensor → ESP32/IoT Node → Low-Power Transmitter → Gateway/Cloud

with:

Sleep → Sense → Transmit → Sleep

Conclusion:
The proposed model improves battery life by minimizing active transmission time and keeping the IoT device in sleep mode whenever communication is not required.

- Design an NB-IoT architecture for smart utility monitoring.

Architecture: Smart Meter → NB-IoT UE → eNB → EPC → IoT Cloud → Utility Monitoring Center

Program Code:-

clc; clear; close all;

% Smart utility monitoring using NB-IoT

N = 10;                 % Number of smart meters

data = randi([1 100],1,N); % Utility readings

Ptx = 200;              % Transmit power (mW)

Ttx = 0.02;             % Transmission time (s)

% Energy used for transmitting each reading

E = Ptx*Ttx;

TotalEnergy = N*E;

fprintf('Number of meters = %d\n',N);

fprintf('Energy per reading = %.2f mJ\n',E);

fprintf('Total energy = %.2f mJ\n',TotalEnergy);

% Display utility readings

figure;

stem(1:N,data,'filled');

xlabel('Smart Meter');

ylabel('Utility Reading');

title('NB-IoT Smart Utility Monitoring');

grid on;

MATLAB Design Output

The program calculates:

- Energy per meter reading

- Total transmission energy

- Utility readings of 10 smart meters

- Graph of monitored utility consumption

Key Design Features

| Feature | NB-IoT Design |
| --- | --- |
| Device | Smart meter |
| Access | NB-IoT |
| Network | eNB + EPC |
| Application | Utility monitoring |
| Power | Low |
| Coverage | Wide |
| Data | Small periodic packets |

Conclusion:
The proposed NB-IoT architecture enables low-power, wide-area and reliable smart utility monitoring, making it suitable for electricity, water and gas meters.