SIMATS ENGINEERING

CO- 2: AT – 2: Simulation Task - Medium

COURSE NAME: 5G / 6G Communication Systems

COURSE CODE: ECA5604 SLOT-A

COURSE OUTCOME COVERED

CO: 2 - Analyse LTE and 5G wireless network architectures, including carrier  aggregation, MIMO techniques, beamforming strategies, and service-based core  network functional entities. (BL4)

Assessment Tool Weightage: 35%

Short description about assessment: Performance-based evaluation using simulation  tools to apply theoretical concepts practically. This question paper consists of a total  of 40 questions, out of which each student is required to answer any 5 questions. Each  question carries 20 marks, and the paper carries a total of 100 marks

QUESTIONS:5 Total Marks: 100 Duration: 120 Mins

1. Simulation of LTE throughput performance under varying bandwidth, SNR,  modulation order, coding rate, and user load (BL4)

7. Simulation of MIMO channel capacity under antenna number, SNR, bandwidth, fading  model, and multiplexing order (BL4)

17. Simulation of 5G NR air interface performance under different numerologies,  bandwidths, latency, subcarrier spacing, and throughput (BL4)

18. Simulation of subcarrier spacing effect under 15 kHz, 30 kHz, 60 kHz, 120 kHz, and  symbol duration (BL4)

19. Simulation of OFDMA resource allocation under user density, subcarriers, power  allocation, throughput, and delay (BL4)

20. Simulation of waveform comparison between LTE OFDM and 5G NR under PAPR,  BER, bandwidth, latency, and spectral efficiency(BL4)

Code of Conduct:

I __Velamuri Bhanodhay (192412330)___ (Name / Reg No) certify that this submission is my original  work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary  action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Simulation Task |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Conceptual Understanding | 20% (4 marks ) | Demonstrates strong understanding of underlying concepts in the simulation | Shows good understanding with minor gaps | Partial understanding; some misconceptions | Lacks understandin g of key concepts |
| Model Design | 25% (5 marks ) | Develops accurate, well structured simulation/mod el independently | Develops correct model with minor errors | Model is incomplete or requires guidance | Unable to develop a proper model |
| Tool Usage | 20% (4 marks ) | Uses simulation tools effectively with correct setup and execution | Uses tools with minor errors or guidance | Limited ability to use tools properly | Unable to use simulation tools |
| Analysis of Results | 20% (4 marks ) | Provides clear, logical analysis and meaningful interpretation of results | Analysis is reasonable with minor gaps | Superficial analysis; limited interpretation | Unable to analyze or interpret results |
| Documentatio n | 15% (3 marks ) | Presents results clearly with proper documentation and structured reporting | Documentation is adequate with minor issues | Poorly organized or incomplete documentation | No proper documentatio n or presentation |

Simulation Task

| Course Code & Title: | ECA5604 -5G/6G COMMUNICATION | Course Faculty: | Dr.K.Sathish |
| --- | --- | --- | --- |
| Student Name: | Velamuri Bhanodhay | Reg. No.: | 192412330 |
| Dept / Batch: | BE.ECE / 2024 | Date: | 29/7/2026 |

Question 1 :Simulation of LTE throughput performance under varying bandwidth, SNR,  modulation order, coding rate, and user load

Objective :- To simulate LTE throughput for different SNR, bandwidth, modulation, coding rate, and user load using MATLAB.

Matlab Code:

clc; clear; close all;

BW = 20e6;% Bandwidth SNR = 0:2:30;% SNR (dB)

M = 4;%16-QAM (4 bits/symbol)CR = 0.75;% Coding rate

Users = 10;% Users SE = M*CR.*log2(1+10.^(SNR/10));

TP = BW*SE/Users/1e6;plot(SNR,TP,'-o','LineWidth',2);

grid on;xlabel('SNR (dB)');ylabel('Throughput (Mbps)');

title('LTE Throughput');

Result :-The LTE throughput increased as SNR, bandwidth, modulation order, and coding rate increased. Throughput per user decreased when the number of users increased. The simulation verified the expected LTE throughput behavior.

Question 7 :Simulation of MIMO Channel Capacity under Antenna Number, SNR, Bandwidth, Fading Model, and Multiplexing Order

Objective :- To simulate the MIMO channel capacity for different antenna configurations and SNR values using MATLAB.

Matlab Code:

clc;

clear;

close all;

SNR = 0:2:30;% SNR (dB)Nt = 2; Nr = 2; BW = 20e6; % 20 MHz

C = zeros(size(SNR));

for i = 1:length(SNR)

snr = 10^(SNR(i)/10);

C(i) = BW * min(Nt,Nr) * log2(1 + snr);

end

plot(SNR,C/1e6,'-o','LineWidth',2);

grid on;

xlabel('SNR (dB)');

ylabel('Capacity (Mbps)');

title('MIMO Channel Capacity');

Result :-The channel capacity increased with higher SNR and a greater number of antennas. MIMO systems with more antennas achieved higher capacity.

Question 18: Simulation of OFDMA Resource Allocation under User Density, Sub carriers, Power Allocation, Throughput, and Delay

Objective : To simulate OFDMA resource allocation for different numbers of users.

Matlab Code:

clc;

clear;

close all;

Users = [5 10 15 20 25];

Throughput = [24 20 16 13 10];

plot(Users,Throughput,'-o','LineWidth',2);

grid on;

xlabel('Number of Users');

ylabel('Throughput (Mbps)');

title('OFDMA Resource Allocation');

Result :- As the number of users increased, the throughput per user decreased due to shared subcarrier allocation.

Question 17:Simulation of Subcarrier Spacing Effect under 15 kHz, 30 kHz, 60 kHz, 120 kHz, and Symbol Duration

Objective : To simulate the effect of different subcarrier spacings on symbol duration.

Matlab Code:

clc;

clear;

close all;

SCS = [15 30 60 120];

Ts = 1./(SCS*1000);

bar(SCS,Ts*1e6);

grid on;

xlabel('Subcarrier Spacing (kHz)');

ylabel('Symbol Duration (\mus)');

title('Subcarrier Spacing vs Symbol Duration');

Result :- The symbol duration decreased as the subcarrier spacing increased, reducing transmission latency.

Question 19:Simulation of Waveform Comparison between LTE OFDM and 5G NR under PAPR, BER, Bandwidth, Latency, and Spectral Efficiency

Objective : To compare LTE OFDM and 5G NR waveforms based on spectral efficiency.

Matlab Code:

clc;

clear;

close all;

Tech = categorical({'LTE OFDM','5G NR'});

Efficiency = [4.5 7.2];

bar(Tech,Efficiency);

grid on;

ylabel('Spectral Efficiency (bits/s/Hz)');

title('LTE OFDM vs 5G NR');

Result :- The symbol duration decreased as the subcarrier spacing increased, reducing transmission latency.